Layer 2 vs. Layer 3 Encryption Testing in High Speed Point-to-Point SONET OC-48 Networks

Luther Troell

Jason Burns

Kurt Chapman

Dave Goddard

Matt Soderlund

Chris Ward

Table of Contents

Executive Summary	3

Introduction	3

Purpose	3

Experimental Design	4

Testing Methodology	4

Topologies	5

Results	6

Throughput	6

Baseline	6

SSE	7

WS-SVC-IPSEC-1	7

Frame Loss	8

Baseline	8

SSE	8

WS-SVC-IPSEC-1	9

Latency	10

Baseline	10

SSE	10

WS-SVC-IPSEC-1	10

Discussion	11

Throughput	11

Frame Loss	11

Latency	11

Recommendations for Further Research	12

Encryption Algorithm	12

IPSec Device Capable of Encrypting OC-48	12

Conclusion	12

Acknowledgement	13

Appendix A – Configuration Examples	14

SONET Baseline and SSE Testing	14

Timmy	14

Jimmy	15

IPSec over SONET with Cisco VPN Accelerator	17

Timmy	17

Jimmy	20

# Executive Summary

For high speed point-to-point links, layer 2 encryption provides an effective solution to secure high speed data networks while minimizing the negative impacts usually associated with encryption. Our testing showed that layer 2 encryption provides higher throughput, lower latency, and less frame loss than layer 3 encryption. The effective utilization of bandwidth makes layer 2 SONET encryption an attractive alternative to the standard layer 3 IPSec encryption.

# Introduction

Many organizations use high speed SONET rings to connect their facilities together. SONET offers high availability, speed, and is a mature technology. In many industries, highly sensitive data travels over those SONET rings. This is especially true in health care and government, but is becoming more important in a wider range of organizations.

Traditionally, customers trusted their carriers with data integrity and authenticity. Unfortunately, relying on outside parties for the integrity and authenticity is not completely secure. Carriers will not be able to physically secure all of their network links.  They simply can’t police oceans or baron fields.  This is a growing problem as tapping into optical network cabling is becoming a simpler task. ZD Net reported that the NSA was working on determining how to tap undersea cables. It is no doubt that other groups are exploring similar technologies. For this reason, it is beneficial to encrypt backbone network links to off site facilities.

Encryption comes at a price. Adding overhead or latency to data traveling over expensive SONET links can be cost prohibitive. An enterprise spending money on high-speed data links may not be willing to sacrifice throughput for security. The cost per megabit of the line can make it hard to justify allocating even a fraction of the link to security related overhead. It is in the best interest of all organizations to find the most efficient solutions to the problem of network security.

# Purpose

The purpose of this set of tests is to determine the performance differences between Layer 2 and Layer 3 encryption technologies over a SONET link. The Layer 2 encryption will be performed by SafeNet SafeEnterprise SONET Encryptors (SSEs), and the Layer 3 encryption will be done with a Cisco VPN Accelerator (WS-SVC-IPSEC-1) to create an IPSec tunnel for testing. The goal is to test and quantify the performance differences between each of the encryption methods.

# Experimental Design

## Testing Methodology

We ran a battery of tests according to the methods outlined in RFC 2544. The three areas of interest for our testing were throughput, frame loss, and latency. These tests allowed for a more complete picture of the performance differences across different encryption technologies than using just sheer throughput as a performance indicator. Many technologies today, like VoIP and streaming media, are sensitive to network latency and frame loss, making these tests very informative.

The throughput test is aimed at finding the maximum possible throughput, in frames per second, across a link with absolutely no frame loss. This test runs multiple iterations for each trial. Each trial starts by sending 100% of the link’s maximum theoretical throughput. The maximum theoretical throughput is calculated in frames per second for each frame size by taking into account the Ethernet preamble and inter-frame gap to find the actual number of frames minus overhead that can be transmitted. If any frames are dropped the test will run again at a lower throughput using a binary search algorithm. After several iterations the highest possible throughput without frame loss will be discovered. Our tests ran with iterations lasting one minute, with ten trials for each of the following frame sizes in bytes: 64, 128, 256, 512, 1024, 1280, and 1420. We chose the previous frame sizes to give a representative sampling from smallest to largest while ensuring that no fragmentation will occur with the addition of IPSec overhead.

The frame loss test determines what percentage of frames is dropped at a certain level of throughput. The frame loss test starts at 100% maximum theoretical throughput for each trial, and then drops by 10% for each iteration within that trial until no frames are dropped. We ran this test with 10,000,000 frames per iteration and 10 trials for each of the following frame sizes in bytes: 64, 128, 256, 512, 1024, 1280, and 1420. This test works well in conjunction with the throughput tests because it allows us to examine the level of frame loss that can be expected when the bandwidth limits for a link are exceeded.

The latency test measures the time in nanoseconds needed for a packet leaving one testing interface on the traffic generator to enter the receiving testing interface. The traffic generator measures this by tagging one frame every second with a time stamp. Because of the way frames are tagged, it is essential that the throughput test be ran first and each latency test is not run above the maximum throughput for that frame size without frame loss. Frame loss during the latency test would interfere with the results. The latency test was run for one minute per trial, with ten trials for each of the following frame sizes in bytes: 64, 128, 256, 512, 1024, 1280, and 1420. In order to make the latency testing fair we calculated the throughput at which the encryption technology ran for a certain frame size, and then ran a latency test at the same percentage and frame size on the baseline setup with no encryption. This allows for a comparison of baseline latency to encrypted latency. The amount of latency added can then be attributed to the encryption device in use.

## Topologies

For the purpose of comparison, we ran tests on three different topologies: Baseline, Cisco IPSec VPN Accelerator, and SafeNet SafeEnterprise SONET Encryptor. The baseline topology consisted of two Cisco Catalyst 6509s connected back to back with a Fiber Optic OC-48 link (Figure 1). On the opposite side of each Catalyst 6509 was an Ixia 400T traffic generator with OC-48 cards was attached to the topology. The Ixia traffic generator was configured to send traffic from each card to the other, allowing us to measure bi-directional performance. The baseline tests were run with no encryption, the clock source set to internal, scrambling enabled, and the CRC set to 32. These tests provide a baseline for comparing the performance of various encryption technologies. Any variance in performance from the baseline can be attributed to the encryption technologies in use.

After the baseline results were gathered, the next topology involved the SafeNet SafeEnterprise SONET Encryptors. Two SSEs were placed in the middle of the connecting OC-48 link and configured to encrypt all traffic passing over the OC-48 line (Figure 2). These devices acted transparently and did not require any changes in configuration on the Catalyst 6509s. The SONET traffic was encrypted with 256 bit AES. There was no option to use the older 3DES encryption on the devices.

The final topology utilized one Cisco WS-SVC-IPSEC-1 VPN Accelerator card in each Catalyst 6509 chassis (Figure 1). This device was configured to encrypt all traffic coming from the Ixia subnets with IPSec using 3DES. 3DES was selected because AES was not an available option on the Cisco VPN accelerator at the time of testing. The routers were configured to use IKE and a Certificate Authority for key exchange. The Security Association lifetime was configured to be one day so that re-keying would not happen during the middle of the tests. We chose a crypto map utilizing esp-3des and sha1-hmac. The Catalyst 6509s were then connected back to back with the traffic to be encrypted traveling through the internal Cisco VPN Accelerator before leaving for the other Catalyst 6509.

Figure 1

Figure 2

# Results

## Throughput

### Baseline

The baseline throughput test between the two back to back routers showed around 50% throughput at smaller frame sizes, but dramatically increased in performance with frames above 256 bytes. At frame sizes of 256 bytes and above the baseline topology was able to achieve 100% of maximum theoretical throughput (Figure 3).

The major loss in throughput at smaller frame sizes can be attributed to the PoS (Packet over SONET) technology in use. Each IP packet the SONET interface receives must be reassembled and sent up the protocol stack for an IP routing decision to be made on that packet. The combination of small IP frame sizes and high speed SONET links leads to a situation where many frames have to be decapsulated, passed up the protocol stack, sent to the right interface, and then encapsulated again. This level of overhead would not be necessary on a true SONET ring, but is inherent to PoS.

### SSE

The SSEs were able to encrypt traffic at the same rate as the baseline test. There was absolutely no difference in throughput between the baseline test and the SafeNet SONET encryption test. The SafeNet devices managed to encrypt an entire OC-48 link with no effect at all on available throughput (Figure 3).

### WS-SVC-IPSEC-1

The VPN Accelerator presented an interesting challenge in testing. The equipment available for testing had a maximum potential throughput of only 1.9Gb/s. To encrypt the full OC-48 link would require 4.8Gb/s. We ran the tests and saw that the VPN Accelerator was able to encrypt 1.9GB/s of traffic, but this was only 39% of the full link.

However, the curve the throughput follows is useful. The VPN Accelerator does not closely follow the IPSec theoretical maximum at all frame sizes. At lower frame sizes the curve of the VPN Accelerator does not rise as fast as the curve of IPSec theoretical maximum (Figure 3).

Figure 3

## Frame Loss

### Baseline

The OC-48 link in the baseline configuration was not able to support 100% of the theoretical maximum throughput, but did not notice extremely severe frame loss when the throughput limit was crossed. The following graph shows that the most frame loss occurred at frame sizes of 64 bytes, with a maximum loss of about 50%.

Figure 4

### SSE

Adding the SafeNet device to the topology had no effect on the percentage of frames dropped.

Figure 5

### WS-SVC-IPSEC-1

Figure 6

As mentioned above, the Cisco IPSec VPN Accelerator was only designed to handle 1.9Gb/s – or 39% of the maximum link.  As a result, high frame loss at more than 39% throughput is to be expected (Figure 6).

## Latency

### Baseline

The baseline topology showed an interesting curve in latency where 64-byte frames experienced higher latency than 128-byte frames. After the frame size reached 128 bytes the curve becomes more normal and exhibits a maximum latency of about 250,000 nanoseconds (Figure 7). This abnormal curve is most likely associated with the previously mentioned limitations of PoS.

### SSE

Adding the SafeNet device to the topology only added a small amount of latency. The SafeNet curve closely resembles the baseline curve with the exception of adding an average of 14,000 nanoseconds of latency (Figure 7). This is a significantly low amount of latency.

### WS-SVC-IPSEC-1

The Cisco VPN Accelerator added a considerable amount of latency to the topology (Figure 7). The accelerator was performing IPSec on the data stream which necessitated encapsulation and decapsulation. This, along with the lower potential throughput of the VPN Accelerator can explain the amount of latency. The VPN Accelerator was being taxed considerably when given the task of encrypting an OC-48 link.

Figure 7

# Discussion

## Throughput

The throughput achieved at smaller frame sizes on the baseline topology was considerably lower than the theoretical maximum. The routers were unable to process the 64 and 128-byte frames at full speed. With frame sizes of 256 bytes and larger, the routers were able to achieve 100% of the maximum theoretical throughput.

Because of these throughput limitations, and the topology, we were unable to determine if the SSEs were capable of higher throughput at lower frame sizes. For our testing, this is acceptable because both encryption technologies were compared to the baseline.

In addition to the limitation of the baseline we were faced with the limitation of the Cisco VPN Accelerator. To give a more detailed picture of potential IPSec performance, we calculated the overhead associated with IPSec and then determined the maximum theoretical throughput of IPSec over SONET. If the VPN accelerator were more capable then it would be likely to approach this limit at larger frame sizes.

It becomes immediately obvious that the performance of the SSEs is already greater than the maximum theoretical throughput that can be reached with IPSec. The overhead that is associated with IPSec in terms of an additional IP header and authentication header means that IPSec can never achieve the performance of SONET encryption with its absolute absence of encryption overhead. This lack of encryption overhead leaves 100% of the SONET line available for data throughput. This is accomplished by using the overhead bits allocated in SONET for transmitting the additional data needed for encryption outside of the data stream.

## Frame Loss

The frame loss seen in the SSE test closely resembles the throughput limitations of the baseline topology. The SSE didn’t add any frame loss, while the VPN Accelerator added considerable frame loss at higher throughput. These numbers are not a very good indicator of performance because of the VPN Accelerator’s 1.9Gb/s throughput limitation.

The only thing that can be concluded from the frame loss tests is that the SSE doesn’t add frame loss, where the VPN Accelerator may if it were capable of higher throughput.

## Latency

In the latency tests the SSE was the better performer. The SSE adds a small and consistent amount of latency to the data being encrypted. The VPN Accelerator adds a much higher level of latency to the encrypted data because of the additional work associated with encapsulating and decapsulating IP packets. The lower latency associated with the SSE’s Layer 2 encryption would be well suited to many real-time applications, such as streaming video or Voice over IP.

# Recommendations for Further Research

## Encryption Algorithm

The Cisco VPN Accelerator utilized 3DES encryption, while the SSE used AES encryption. Unfortunately, we were unable to find a Cisco VPN Accelerator that supported AES. Since we were only able to judge the Cisco equipment based on 3DES, the test is not perfectly controlled. We believe the differences between the two encryption technologies would not significantly alter the test results, but have no empirical evidence with this exact equipment. The differences in algorithm may have an effect, the extent of which we have no data from these tests to determine.

## IPSec Device Capable of Encrypting OC-48

The Cisco VPN Accelerator’s limitations on throughput are an interesting challenge. Currently three VPN accelerators would have to be purchased for each Catalyst 6509 to encrypt an entire OC-48 line. With this hardware setup it would be possible to determine if the VPN Accelerator was able to achieve the maximum theoretical throughput of IPSec. This is purely for scientific reasons. Even if the IPSec solution were able to perform at its theoretical max, it would still be slower than the layer 2 encryption solution.

# Conclusion

To securely connect a high speed point-to-point SONET link, layer 2 encryption provides better performance all around. Layer 2 encryption with the SafeNet SafeEnterprise SONET Encryptor provided greater throughput, less frame loss, and lower latency than layer 3 encryption with the Cisco VPN Accelerator.

The layer 2 SSE was able to fully utilize the expensive bandwidth of an OC-48 line. The layer 3 IPSec solution is not able to reach this level of utilization in theory, nor in practice. Because layer 2 encryption adds no overhead to the data stream it is able to provide the maximum level of data throughput.

Layer 2 encryption adds less latency than layer 3 encryption. This makes layer 2 encryption a more attractive choice for networks with delay sensitive streaming applications running over them like VoIP and real-time conferences.

Layer 2 encryption is able to achieve higher throughput, and thus adds no frame loss at larger frame sizes. Layer 3 encryption solutions can become oversaturated, and drop frames, possibly requiring time costly retransmission of data. The SSE transmitted more data per unit time and did so with greater efficiency and reliability.

For a point-to-point link, layer 2 encryption is the ideal solution. The expensive bandwidth is effectively utilized while minimizing all of the negative impacts usually associated with adding encryption. The types of performance degradation typically seen with layer 3 encryption are considerably lessened or eliminated by using layer 2 encryption.

# Acknowledgement

We would like to thank Ixia Corporation for the generous loan of the test generation and measurement equipment used in this study.  This consisted of the Ixia 400 chassis, Encryption Load Module, OC-48 PoS Load Module, Ethernet Load Module, and corresponding software (IxCharriot, IxExplorer and IxVPN). The use of these Ixia products provided significant benefit in our test execution and data gathering efforts.  It would not have been possible to perform this study in the three-month timeframe without use of this equipment and on-site assistance provided by Ixia.  We are deeply appreciative of their support.

Appendix A – Configuration Examples

## SONET Baseline and SSE Testing

### Timmy

!

version 12.2

service timestamps debug uptime

service timestamps log uptime

service password-encryption

service counters max age 10

!

hostname timmy

!

boot system flash s72033-pk9sv-mz.122-18.SXD3.bin

enable secret 5 $1$3kha$v7M6Xr2wK9XkbnoqOAMqc/

!

no aaa new-model

clock timezone EST -5

ip subnet-zero

!

!

ip domain-name sonetbonnet.com

ip host jimmy 172.16.1.12

ip host smcmgmt.sonetbonnet.com 172.16.1.23

!

mls ip multicast flow-stat-timer 9

no mls flow ip

no mls flow ipv6

mls cef error action freeze

!

!

power redundancy-mode combined

spanning-tree mode pvst

no spanning-tree optimize bpdu transmission

diagnostic cns publish cisco.cns.device.diag_results

diagnostic cns subscribe cisco.cns.device.diag_commands

!

redundancy

mode sso

main-cpu

auto-sync running-config

!

vlan internal allocation policy ascending

vlan access-log ratelimit 2000

!

!

!

interface POS2/1

ip address 10.10.10.1 255.255.255.0

no keepalive

mls qos trust dscp

crc 32

clock source internal

pos scramble-atm

pos flag c2 22

!

interface POS2/2

ip address 192.168.101.254 255.255.255.0

no keepalive

mls qos trust dscp

crc 32

clock source internal

pos scramble-atm

pos flag c2 22

!

interface FastEthernet3/1

ip address 172.16.1.11 255.255.255.0

!

!

interface GigabitEthernet5/1

no ip address

shutdown

!

interface GigabitEthernet5/2

no ip address

shutdown

!

interface Vlan1

no ip address

shutdown

!

!

ip classless

ip route 192.168.201.0 255.255.255.0 10.10.10.2

no ip http server

!

!

!

access-list 101 permit ip 192.168.101.0 0.0.0.255 192.168.201.0 0.0.0.255

!

!

!

control-plane

!

!

!

line con 0

line vty 0 4

password 7 111A160B120609030A242E30

login

!

!

ntp clock-period 17181487

ntp server 172.16.1.23

end

### Jimmy

!

version 12.2

service timestamps debug uptime

service timestamps log uptime

service password-encryption

service counters max age 10

!

hostname jimmy

!

boot system flash s72033-pk9sv-mz.122-18.SXD3.bin

enable secret 5 $1$/ql/$AjgJQiTG4wufJ8p/DdBqM1

!

no aaa new-model

clock timezone EST -5

ip subnet-zero

!

!

ip domain-name sonetbonnet.com

ip host timmy 172.16.1.11

ip host smcmgmt 172.16.1.23

!

mls ip multicast flow-stat-timer 9

no mls flow ip

no mls flow ipv6

mls cef error action freeze

!

!

power redundancy-mode combined

spanning-tree mode pvst

no spanning-tree optimize bpdu transmission

diagnostic cns publish cisco.cns.device.diag_results

diagnostic cns subscribe cisco.cns.device.diag_commands

!

redundancy

mode sso

main-cpu

auto-sync running-config

!

vlan internal allocation policy ascending

vlan access-log ratelimit 2000

!

!

interface POS2/1

ip address 10.10.10.2 255.255.255.0

no keepalive

mls qos trust dscp

crc 32

clock source internal

pos scramble-atm

pos flag c2 22

!

interface POS2/2

ip address 192.168.201.254 255.255.255.0

no keepalive

mls qos trust dscp

crc 32

clock source internal

pos scramble-atm

pos flag c2 22

!

interface FastEthernet3/1

ip address 172.16.1.12 255.255.255.0

!

!

interface GigabitEthernet5/1

no ip address

shutdown

!

interface GigabitEthernet5/2

no ip address

shutdown

!

interface Vlan1

no ip address

shutdown

!

!

ip classless

ip route 192.168.101.0 255.255.255.0 10.10.10.1

no ip http server

!

!

!

control-plane

!

!

!

line con 0

line vty 0 4

password 7 083243400C0D07181C050910

login

!

!

ntp authenticate

ntp clock-period 17181374

ntp server 172.16.1.23

end

## IPSec over SONET with Cisco VPN Accelerator

### Timmy

!

version 12.2

service timestamps debug uptime

service timestamps log uptime

no service password-encryption

service counters max age 10

!

hostname Timmy

!

boot system flash bootflash:s72033-pk9sv-mz.122-18.SXD3.bin

boot system flash bootflash:

enable secret 5 $1$PFW1$8BBnI3NrSPGbyDOVDiLc9.

!

no aaa new-model

ip subnet-zero

!

!

ip domain-name sonetbonnet.com

ip host jimmy 172.16.1.12

ip host smcmgmt.sonetbonnet.com 172.16.1.23

!

mls ip multicast flow-stat-timer 9

no mls flow ip

no mls flow ipv6

mls cef error action freeze

!

crypto ca trustpoint smcmgmt.sonetbonnet.com

enrollment profile timmyCAprofile

serial-number

fqdn Timmy.sonetbonnet.com

ip-address 10.10.10.1

subject-name C=US, ST=New York, L=Rochester, O=RIT, OU=IT, CN=Timmy

revocation-check none

rsakeypair timmykey

!

crypto ca profile enrollment timmyCAprofile

enrollment url  http://smcmgmt.sonetbonnet.com:80/cgi-bin/pkiclient.exe

!

!

crypto ca certificate chain smcmgmt.sonetbonnet.com

certificate 1E

3082028C 308201F5 02011E30 0D06092A 864886F7 0D010105 05003071 310B3009

06035504 06130255 53311130 0F060355 04081308 4E657720 596F726B 31123010

06035504 07130952 6F636865 73746572 310C300A 06035504 0A130352 4954310B

30090603 55040B13 02495431 20301E06 03550403 13174365 72746966 69636174

696F6E20 41757468 6F726974 79301E17 0D303530 34323231 38343933 305A170D

33303034 32323138 34393330 5A3081AB 310E300C 06035504 03130554 696D6D79

310B3009 06035504 0B130249 54310C30 0A060355 040A1303 52495431 12301006

03550407 1309526F 63686573 74657231 11300F06 03550408 13084E65 7720596F

726B310B 30090603 55040613 02555331 4A300B06 03550405 13044136 44313017

06092A86 4886F70D 01090813 0A31302E 31302E31 302E3130 2206092A 864886F7

0D010902 16155469 6D6D792E 736F6E65 74626F6E 6E65742E 636F6D30 819F300D

06092A86 4886F70D 01010105 0003818D 00308189 02818100 C6B3651F 3A07E55F

A1C75663 58D6A809 C1FD58A2 7FD7182C A4EAD83F CF0405ED 0B86BC81 1988085C

FB6B5AD3 D7A4F040 488E4ED2 221F9FFA 9B6B773B 55BA1717 DCBC8739 CDB6A8D2

813D63ED EFC2FB81 A2CE33ED 7D16D69C 2C2FB788 9BB0E96B 86B1EC83 6C9EE86F

C78ADF5A C5E3606B D35650F4 82BF35A6 8054462F 767DAEB5 02030100 01300D06

092A8648 86F70D01 01050500 03818100 28CB926B 322E719D 334A3CF2 605D0A20

0FE97667 66810442 33717CA2 3BEE4F2E 7F5317FA 202A7D62 D46E3FBA B39D8D8B

81746667 F830D63A E5E4FF14 8C412099 316653B4 353E6431 590A7E65 C9C6CED2

9DA064B0 5BC08682 2D0695D7 890DBAE7 947EBEAA 02C1E15A 7C30E21E 936B7159

C14C88D1 24012AF0 6D5F3ED3 3BC62598

quit

certificate ca 02

30820251 308201BA 02010230 0D06092A 864886F7 0D010105 05003071 310B3009

06035504 06130255 53311130 0F060355 04081308 4E657720 596F726B 31123010

06035504 07130952 6F636865 73746572 310C300A 06035504 0A130352 4954310B

30090603 55040B13 02495431 20301E06 03550403 13174365 72746966 69636174

696F6E20 41757468 6F726974 79301E17 0D303530 33313531 37343130 365A170D

33303033 31353137 34313036 5A307131 0B300906 03550406 13025553 3111300F

06035504 0813084E 65772059 6F726B31 12301006 03550407 1309526F 63686573

74657231 0C300A06 0355040A 13035249 54310B30 09060355 040B1302 49543120

301E0603 55040313 17436572 74696669 63617469 6F6E2041 7574686F 72697479

30819F30 0D06092A 864886F7 0D010101 05000381 8D003081 89028181 00AB9F12

59352433 6356CD84 DF59F059 9EAD3A41 DED50FEA 13D08BB9 220A6103 9F27F071

5E958954 6145EFE1 B5102A55 A9EBF988 C950E4AC 650BA38C 48DF9373 DB420AE8

E1C381A1 1DDBCBFF 2F85920E 3FCC1EC4 E531C4B1 05100E52 702D0E4D 29681DEE

9B84D7F1 AF0A973A F1276A94 9E17BF98 1B0C8EF4 38BFB332 C31014FF 71020301

0001300D 06092A86 4886F70D 01010505 00038181 001F744F DCB6BB42 675BB031

CFB282AD 3F730210 10431901 4B47F4D7 0612D5B6 62924153 53840C10 664BC757

81BF4288 3B4E34C5 A648D8AA 7B3F693C 9B58AA07 A26011AA 0AD93218 4DB59993

864AB9AE 58402490 F3B433A0 33EE6ED9 74965563 1132752D DBFDA24F C90601CE

0B1E1691 0AAA83D3 8758F0D7 987FE2DF F9A2C7AD 7D

quit

!

!

crypto isakmp policy 100

encr 3des

group 2

crypto isakmp profile timmyprofile

ca trust-point smcmgmt.sonetbonnet.com

match identity address 10.10.10.2 255.255.255.255

!

crypto ipsec security-association lifetime kilobytes 536870912

!

crypto ipsec transform-set set1 esp-3des esp-sha-hmac

!

crypto map toJimmy 10 ipsec-isakmp

set peer 10.10.10.2

set transform-set set1

match address 101

!

!

power redundancy-mode combined

spanning-tree mode pvst

no spanning-tree optimize bpdu transmission

diagnostic cns publish cisco.cns.device.diag_results

diagnostic cns subscribe cisco.cns.device.diag_commands

!

redundancy

mode sso

main-cpu

auto-sync running-config

!

vlan internal allocation policy ascending

vlan access-log ratelimit 2000

!

!

interface GigabitEthernet1/1

no ip address

flowcontrol receive on

flowcontrol send off

switchport

switchport trunk encapsulation dot1q

switchport trunk allowed vlan 1,102,1002-1005

switchport mode trunk

spanning-tree portfast trunk

!

interface GigabitEthernet1/2

no ip address

flowcontrol receive on

flowcontrol send off

switchport

switchport trunk encapsulation dot1q

switchport trunk allowed vlan 1,1002-1005

switchport mode trunk

spanning-tree portfast trunk

!

interface POS2/1

no ip address

no keepalive

mls qos trust dscp

crc 32

clock source internal

pos scramble-atm

pos flag c2 22

crypto connect vlan 102

!

interface POS2/2

ip address 192.168.101.254 255.255.255.0

no keepalive

mls qos trust dscp

crc 32

clock source internal

pos scramble-atm

pos flag c2 22

!

interface FastEthernet3/1

no ip address

switchport

switchport access vlan 172

switchport mode access

!

!

interface GigabitEthernet5/1

no ip address

shutdown

!

interface GigabitEthernet5/2

no ip address

shutdown

!

interface Vlan1

no ip address

shutdown

!

!

interface Vlan102

ip address 10.10.10.1 255.255.255.0

crypto map toJimmy

crypto engine slot 1

!

interface Vlan172

ip address 172.16.1.11 255.255.255.0

!

ip classless

ip route 192.168.201.0 255.255.255.0 10.10.10.2

no ip http server

!

!

!

access-list 101 permit ip 192.168.101.0 0.0.0.255 192.168.201.0 0.0.0.255

!

!

!

control-plane

!

!

!

line con 0

line vty 0 4

password 7 111A160B120609030A242E30

login

!

!

end

### Jimmy

!

version 12.2

service timestamps debug uptime

service timestamps log uptime

service password-encryption

service counters max age 10

!

hostname Jimmy

!

boot system bootflash:s72033-pk9sv-mz.122-18.SXD3.bin

boot system bootflash:

enable secret 5 $1$ASW1$CbDspCvW1uQqW6oqmxIYj1

!

no aaa new-model

ip subnet-zero

!

!

ip domain-name sonetbonnet.com

ip host timmy 172.16.1.11

ip host smcmgmt.sonetbonnet.com 172.16.1.23

!

mls ip multicast flow-stat-timer 9

no mls flow ip

no mls flow ipv6

mls cef error action freeze

!

crypto ca trustpoint smcmgmt.sonetbonnet.com

enrollment profile jimmyCAprofile

serial-number

fqdn Jimmy.sonetbonnet.com

ip-address 10.10.10.2

subject-name C=US, ST=New York, L=Rochester, O=RIT, OU=IT, CN=Jimmy

revocation-check none

rsakeypair jimmykey

!

crypto ca profile enrollment jimmyCAprofile

enrollment url  http://smcmgmt.sonetbonnet.com:80/cgi-bin/pkiclient.exe

!

!

crypto ca certificate chain smcmgmt.sonetbonnet.com

certificate 1F

3082028C 308201F5 02011F30 0D06092A 864886F7 0D010105 05003071 310B3009

06035504 06130255 53311130 0F060355 04081308 4E657720 596F726B 31123010

06035504 07130952 6F636865 73746572 310C300A 06035504 0A130352 4954310B

30090603 55040B13 02495431 20301E06 03550403 13174365 72746966 69636174

696F6E20 41757468 6F726974 79301E17 0D303530 34323231 38343935 395A170D

33303034 32323138 34393539 5A3081AB 310E300C 06035504 0313054A 696D6D79

310B3009 06035504 0B130249 54310C30 0A060355 040A1303 52495431 12301006

03550407 1309526F 63686573 74657231 11300F06 03550408 13084E65 7720596F

726B310B 30090603 55040613 02555331 4A300B06 03550405 13043743 39433017

06092A86 4886F70D 01090813 0A31302E 31302E31 302E3230 2206092A 864886F7

0D010902 16154A69 6D6D792E 736F6E65 74626F6E 6E65742E 636F6D30 819F300D

06092A86 4886F70D 01010105 0003818D 00308189 02818100 9ABB9654 74E1E370

03EEDBB0 F0A72BFF 698A521E 9F3F441D 10DAC4EC 4DE926E4 671AB422 B8C169F3

FD76F3A6 D1D852A7 15D39DAF BEF03287 EAF51320 8A477AB2 DE9E32A6 B71F128A

B0A3943A 4BD44606 5ED3D1AC 85A99920 C6F41B50 C76FB905 DEBE8ABF BDDB267B

A46EEA2E 93E8FF34 A5F2C331 E9E67688 BAB4B276 5E0DC615 02030100 01300D06

092A8648 86F70D01 01050500 03818100 1D0047E7 894D3596 4DDDB1DC 5A549EB2

3785FE1D 11E368DA 4A12228F 49BC8658 20030C5E DA4CBFCB C7C30D1B F351B31F

6C0AE20F 9CBD4D53 119E243F C72A6A00 CA3E5EEA 5CC70446 EF6E71AA 63E092ED

4BEAD927 C3A10A05 B8C6ABA6 6201BFA0 046AEB61 F429BAEE 5F536E5B 9E278B9B

CA922967 D10E8342 E6863B20 6B9BD3C5

quit

certificate ca 02

30820251 308201BA 02010230 0D06092A 864886F7 0D010105 05003071 310B3009

06035504 06130255 53311130 0F060355 04081308 4E657720 596F726B 31123010

06035504 07130952 6F636865 73746572 310C300A 06035504 0A130352 4954310B

30090603 55040B13 02495431 20301E06 03550403 13174365 72746966 69636174

696F6E20 41757468 6F726974 79301E17 0D303530 33313531 37343130 365A170D

33303033 31353137 34313036 5A307131 0B300906 03550406 13025553 3111300F

06035504 0813084E 65772059 6F726B31 12301006 03550407 1309526F 63686573

74657231 0C300A06 0355040A 13035249 54310B30 09060355 040B1302 49543120

301E0603 55040313 17436572 74696669 63617469 6F6E2041 7574686F 72697479

30819F30 0D06092A 864886F7 0D010101 05000381 8D003081 89028181 00AB9F12

59352433 6356CD84 DF59F059 9EAD3A41 DED50FEA 13D08BB9 220A6103 9F27F071

5E958954 6145EFE1 B5102A55 A9EBF988 C950E4AC 650BA38C 48DF9373 DB420AE8

E1C381A1 1DDBCBFF 2F85920E 3FCC1EC4 E531C4B1 05100E52 702D0E4D 29681DEE

9B84D7F1 AF0A973A F1276A94 9E17BF98 1B0C8EF4 38BFB332 C31014FF 71020301

0001300D 06092A86 4886F70D 01010505 00038181 001F744F DCB6BB42 675BB031

CFB282AD 3F730210 10431901 4B47F4D7 0612D5B6 62924153 53840C10 664BC757

81BF4288 3B4E34C5 A648D8AA 7B3F693C 9B58AA07 A26011AA 0AD93218 4DB59993

864AB9AE 58402490 F3B433A0 33EE6ED9 74965563 1132752D DBFDA24F C90601CE

0B1E1691 0AAA83D3 8758F0D7 987FE2DF F9A2C7AD 7D

quit

!

!

crypto isakmp policy 100

encr 3des

group 2

crypto isakmp profile jimmyprofile

ca trust-point smcmgmt.sonetbonnet.com

match identity address 10.10.10.1 255.255.255.255

!

crypto ipsec security-association lifetime kilobytes 536870912

!

crypto ipsec transform-set set1 esp-3des esp-sha-hmac

!

crypto map toTimmy 10 ipsec-isakmp

set peer 10.10.10.1

set transform-set set1

match address 101

!

!

power redundancy-mode combined

spanning-tree mode pvst

no spanning-tree optimize bpdu transmission

diagnostic cns publish cisco.cns.device.diag_results

diagnostic cns subscribe cisco.cns.device.diag_commands

!

redundancy

mode sso

main-cpu

auto-sync running-config

!

vlan internal allocation policy ascending

vlan access-log ratelimit 2000

!

!

interface GigabitEthernet1/1

no ip address

flowcontrol receive on

flowcontrol send off

switchport

switchport trunk encapsulation dot1q

switchport trunk allowed vlan 1,102,1002-1005

switchport mode trunk

spanning-tree portfast trunk

!

interface GigabitEthernet1/2

no ip address

flowcontrol receive on

flowcontrol send off

switchport

switchport trunk encapsulation dot1q

switchport trunk allowed vlan 1,1002-1005

switchport mode trunk

spanning-tree portfast trunk

!

!

interface POS2/1

no ip address

no keepalive

mls qos trust dscp

crc 32

clock source internal

pos scramble-atm

pos flag c2 22

crypto connect vlan 102

!

interface POS2/2

ip address 192.168.201.254 255.255.255.0

no keepalive

mls qos trust dscp

crc 32

clock source internal

pos scramble-atm

pos flag c2 22

!

interface FastEthernet3/1

no ip address

switchport

switchport access vlan 172

switchport mode access

!

!

interface GigabitEthernet5/1

no ip address

shutdown

!

interface GigabitEthernet5/2

no ip address

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan102

ip address 10.10.10.2 255.255.255.0

crypto map toTimmy

crypto engine slot 1

!

interface Vlan172

ip address 172.16.1.12 255.255.255.0

!

!

ip classless

ip route 192.168.101.0 255.255.255.0 10.10.10.1

no ip http server

!

!

!

access-list 101 permit ip 192.168.201.0 0.0.0.255 192.168.101.0 0.0.0.255

!

!

!

control-plane

!

!

!

line con 0

line vty 0 4

password 7 1316181C0E180625252A2D27

login

!

!

end

http://news.zdnet.com/2100-9595_22-529826.html?legacy=zdnn